| Your logo | Your company name Address Phone Email License # / registration # | TOOLBOX TALK: FALL PROTECTION Date Job address Crew lead |
| --- | --- | --- |

TODAY'S TALK

Fill in: the header and the rescue plan before the talk. Then read each line aloud.

1.	OSHA, 29 CFR 1926.501(b)(13): “Each employee engaged in residential construction activities 6 feet (1.8 m) or more above lower levels shall be protected by guardrail systems, safety net system, or personal fall arrest system …” In plain words: anyone 6 feet or more above a lower level needs fall protection. That is the roof edge and the open deck.

2.	Harness on, lanyard clipped to a rated anchor, before you step on the roof. Check the anchor is screwed to a rafter, not just the decking.

3.	Inspect your harness and lanyard now: webbing cuts, bent hooks, stitching. Tag out anything doubtful.

4.	Ladder: 4 to 1 angle, 3 feet above the edge, tied off, feet on solid ground.

5.	Keep the deck clear. Loose shingles, cords and tools cause slips.

6.	Wet, frost or high wind: stop and tell the lead.

7.	Skylights and holes get a cover or a guardrail before anyone works near them.

8.	If someone falls and hangs, call 911 first, then start the rescue plan: ______________________________________ (fill in before the talk)

SITE CHECK TODAY

☐ Anchors installed     ☐ Harnesses inspected     ☐ Ladders tied off     ☐ Holes and skylights covered

☐ Weather OK     ☐ First-aid kit on site     ☐ Rescue plan known

SIGN-IN

| Name (print) | Signature |
| --- | --- |

Lead signature 		Date